| JST-XA Crimp Terminal Fem. | JST-XA Crimp Terminal Fem. |  |  |
| --- | --- | --- | --- |
| MPN: SXA-01T-P0.6 | MPN: SXA-001T-P0.6 |  |  |
| Series = XA V = 250 V I = 3 A Wire Gauge = 20 to 24 AWG Insulation O.D. = 1.5 to 1.9 mm (0.22 – 0.5 mm2) Material = Phosphor Bronze Plating = Tin-plated (Reflow treatment) Extras = RoHS2 Compliance | Series = XA V = 250 V I = 3 A Wire Gauge = 22 to 28 AWG Insulation O.D. = 0.8 to 1.9 mm (0.08 – 0.33 mm2) Material = Phosphor Bronze Plating = Tin-plated (Reflow treatment) Extras = RoHS2 Compliance |  |  |
| JST-XA 2 Circuits Male | JST-XA 2 Circuits Male | JST-XA 3 Circuits Male | JST-XA 4 Circuits Male |
| MPN: B02B-XASK-1 | MPN: B02B-XASK-1N | MPN: B03B-XASK-1 | MPN: B04B-XASK-1 |
| Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Shrouded Header Without a Boss Material = Glass-filled PA 66, Copper A.,Tin-plated Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (Ivory) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Shrouded Header Without a Boss Material = Glass-filled PA 66, Copper A.,Tin-plated Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (White) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Shrouded Header Without a Boss Material = Glass-filled PA 66, Copper A.,Tin-plated Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (Ivory) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Shrouded Header Without a Boss Material = Glass-filled PA 66, Copper A.,Tin-plated Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (Ivory) |
| JST-XA 4 Circuits Male | JST-XA 5 Circuits Male |  |  |
| MPN: S04B-XASK-1 | MPN: B05B-XASK-1 |  |  |
| Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Side Entry with a Hook Material = Glass-filled PA 66, Copper A.,Tin-plated Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (Ivory) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Shrouded Header Without a Boss Material = Glass-filled PA 66, Copper A.,Tin-plated Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (Ivory) |  |  |
| JST-XA 2 Circuits Fem. | JST-XA 3 Circuits Fem. | JST-XA 4 Circuits Fem. | JST-XA 5 Circuits Fem. |
| MPN: XARP-02V | MPN: XARP-03V | MPN: XARP-04V | MPN: XARP-05V |
| Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Housing Retainer Mountable Type Material = Polyamide (PA 66), Nylon 6/6 Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (White) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Housing Retainer Mountable Type Material = Polyamide (PA 66), Nylon 6/6 Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (White) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Housing Retainer Mountable Type Material = Polyamide (PA 66), Nylon 6/6 Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (White) | Series = XA Pitch = 2.5 mm Voltage = 250 V Current = 3 A Type = Housing Retainer Mountable Type Material = Polyamide (PA 66), Nylon 6/6 Temperature Rating = -25 to 85° C Extras: RoHS2, UL 94V-0, Natural (White) |

Some pictures for the connectors are available here: https://gam-gec.com/product/xarp-05v/