| LM741CN/NOPB | MCP6242-E/P | LM358AN/NOPB | LM386N-1/NOPB |
| --- | --- | --- | --- |
| General Purpose Op-Amp | 2 Circuit Rail-to-Rail Op Amp | 2 Circuit General P. Op-Amp | Low Voltage Audio Pw. Amplifier |
| VS = ±15 V Nom. IS = 2.8 mA Max VDiff. In = ±30 V Max IIB = 80 nA Typ. VOS = 2 mV Typ. IOS = 20 mA Typ. IOut. Short. C. = 25 mA AV = 200 V/Vm Typ. CMMR = 90 dB Typ. RIn = 2 MΩ Typ. TOP = 0 to 70° C RθJA = 100° C/W | VDD = 1.8 to 5.5 V ISC = ±6 to ±23 mA IQ = 50 µA Typ. IB = ±1 pA Typ. VOS = ±5 mV IOS = ±1 pA Typ. GBWP = 550 kHz AOL = 110 dB Typ. CMMR = 75 dB Typ. TA = -40 to 125° C θJA = 85° C/W | VS(single) = 3 to 32 V ISC = 40 mA Typ. IS = 1 mA Typ. IB = 45 nA Typ. VOS = 2 mV Typ. IOS = 5 nA Typ. GBW = 1 MHz AV = 100 V/Vm Typ. CMMR = 85 dB Typ. TA = 0 to 70° C RθJA = 120° C/W | VCC = 4 to 12 V ISC = 40 mA Typ. IQ = 4 mA Typ. IBias = 250 nA AV (Vs=6V, f=1kHz)= 26 dB RIN = 50 Ω BW = 300 kHz THD = 0.2% POUT = 325 mW Typ. PSRR = 50 dB TA = 0 to 70° C RθJA = 53° C/W |
| 24LC256-I/P | 24LC1026-I/P | 25LC256-I/P | ATTINY85-20PU |
| 256-Kbit I2C Serial EEPROM | 1024-Kbit I2C CMOS Serial EEPROM | 256-Kbit SPI Bus Serial EEPROM | 8-bit MCU, 8kB Flash 6 I/O Pins |
| VCC = 2.5 to 5.5 V VOL = 0.4 V Max FCLCK = 400 kHz Max VHYS = 0.05 VCC Min VIH = 0.7 VCC Min VIL = 0.3 VCC Max ICC Read = 400 µA ICC Write = 3 mA ICCS = 1.5 µA Max ESD Protc. = ≥4kV TOP = -40 to 85° C | VCC = 2.5 to 5.5 V VOL = 0.4 V Max FCLCK = 400 kHz Max VHYS = 0.05 VCC Min VIH = 0.7 VCC Min VIL = 0.3 VCC Max ICC Read = 450 µA ICC Write = 5 mA ICCS = 5 µA Max ESD Protc. = ≥4kV TOP = -40 to 85° C | VCC = 2.5 to 5.5 V VOL = 0.4 V Max FCLCK = 10 MHz Max VOL = VCC -0.5 V Min VIH = 0.7 VCC Min VIL = 0.3 VCC Max ICC Read = 6 mA ICC Write = 5 mA ICCS = 1 µA Max ESD Protc. = 4kV TOP = -40 to 125° C | VCC = 2.7 to 5.5 V FCLCK = 20 MHz Max Flash = 8 KB SRAM = 256 B TOP = -40 to 85° C EEPROM = 512 B DC Current per I/O Pin = 40 mA Max RPU (I/O Pull-up Res.) = 20 kΩ Min, 50 kΩ Max Interfaces: USI, debugWIRE, SPI |
| CD4017BE | SN74AHCT14N | SN74HC595N | L293DNE |
| 5-stage Johnson Decade Counter | Hex Schmitt-Trigger Inverter | 8-bit Shift Register w/3-State Outp. R. | 600 mA Quadruple Half-H Drivers |
| VDD = 3 to 18 V fCL (clock inp. frequency) at 5 V= 2.5 MHz Max tW (clock pulse width) at 5 V = 200 ns Min IOL Min at 5 V = 1 mA IOL Min at 5 V = -1 mA IDD Max (quiescent dev. curr.) at 5V = 0.04 µA Max TA = -55 to 125° C | VCC = 4.5 to 5.5 V VI = 0 to 5.5 V Max VO = 0.3 to VCC IOH = -8 mA Max IOL = 8 mA Max VOH = 3.8 V Min VOL = 0.44 V Max ΔVT = 1.5 V Max TOP = -40 to 125° C RθJA = 61.1° C/W | VCC = 2 to 6 V IO = ±45 mA Max fclock = 25 MHz Max VI = 0 to VCC VO = 0 to VCC VIH = 3.15 V Min VIL = 1.35 V Max Δt/Δv = 500 ns/V Max TOP = -40 to 85° C RθJA = 67° C/W | VCC1 = 4.5 to 7 V VCC2 = VCC1 to 36 V IO = -600 mA / 600mA Min/Max VIH = 2.3 to 7 V VIL = -0.3 to 1.5 V RθJA = 36.4° C/W TA = 0 to 70° C |
| SN754410NE | ATMEGA328P-PN | ATMEGA168PA-PN | TXS0102DQMR |
| 1 A Quadruple Half-H Driver | 8-bit MCU, 32KB Flash 23 I/O Pins | 8-bit MCU, 16KB Flash 23 I/O Pins | 2-Bit Bidirect. Volt.-Level Translator |
| VCC1 = 4.5 to 5.5 V VCC2 = 4.5 to 36 V IO = ±1 A Max VIH = 2 to 5.5 V VIL = -0.3 to 0.8 V RθJA = 60° C/W TA = -40 to 85° C | VCC = 1.8 to 5.5 V FCLCK = 20 MHz Max Flash = 32 KB SRAM = 2 KB TOP = -40 to 105° C EEPROM = 1 KB DC Current per I/O Pin = 40 mA Max RPU (I/O Pull-up Res.) = 20 kΩ Min, 50 kΩ Max Interfaces: USART, I2C, SPI, ADC, PWM, debugWIRE | VCC = 1.8 to 5.5 V FCLCK = 20 MHz Max Flash = 16 KB SRAM = 1 KB TOP = -40 to 105° C EEPROM = 512 B DC Current per I/O Pin = 40 mA Max RPU (I/O Pull-up Res.) = 20 kΩ Min, 50 kΩ Max Interfaces: USART, I2C, SPI, ADC, PWM, debugWIRE | VCCA = 1.65 to 3.6 V VCCB = 2.3 to 5.5 V VIH = VCC – 0.4 to VCCI VIL = 0 to 0.15 V VOHA = VCCA x 0.67 V VOLA = 0.4 V VOHB = VCCB x 0.67 V VOLB = 0.4 V Δt/Δv = 10 ns/V Max TOP = -40 to 85° C RθJA = 239° C/W |
| BV1HD090FJ-CE2 | BV1LE080EFJ-CE2 | BM2LE080FJ-CE2 | FODM217DR2V |
| 1 Channel High-Side Switch | 1 Channel Low-Side Switch | 2 Channel Low-Side Switch | 1 Channel Phototransis. Octocoupler |
| VBB = 4.5 to 36 V IOUT = 9 A VIN = -0.3 to 7 V EAS at 25° C = 242 mJ IST = 10 mA VINHYS = 0.4 V Typ. VINH = 2.8 V Min VINL = 1.5 V Max IINH = 150 µA Max IINL = ±150 µA TJ = -40 to 150° C θJA = 147° C/W | VOUT = -0.3 to 40 V IOUT(LIM) = 13 A Typ. VIN = 3 to 5.5 V EAS at 25° C = 200 mJ IST = 10 mA RDS(ON) = 80 mΩ Typ. VINH(TH) = 3 V Max IINH1 = 200 µA Max IINL = ±10 µA TJ = -40 to 150° C θJA = 131° C/W | VOUT = -0.3 to 40 V IOUT = 9 A VIN = 3 to 5.5 V EAS at 25° C = 200 mJ IST = 10 mA RDS(ON) = 80 mΩ Typ. VIN(TH) = 3 V Max IINH1 = 200 µA Max IINL = ±10 µA TJ = -40 to 150° C θJA = 141° C/W | BVCEO = 80 V Min IC(AVG) = 50 mA Max BVECO = 7 V Min VISO = 3750 VACRMS VF(Emitter) = 1.2 V Typ. IR(Emitter) = 10 µA Max VR(Emitter) = 6 V Typ. IF(AVG) = 50 mA Max CTRCE = 300 to 600% CT 1 kHz = 30 pF Typ. TOPR = -55 to 110° C |

| 140356145400 | TL071CP | LM324N | NE5532P |
| --- | --- | --- | --- |
| 1 Channel Phototransis. Octocoupler | Low-Noise FET-Input Op Amp | Gen. Purpose Quadruple Op Amp | Dual Low-Noise Op Amp |
| VCE = 80 V Max ICE.P = 50 mA Max VEC = 7 V Min VISO = 3750 V (RMS) VF(Emitter) = 1.24 V Typ. IR(Emitter) = 10 µA Max VR(Emitter) = 6 V Max IF(MAX) = 60 mA Max CTRCE = 300 to 500% Cin 1 kHz = 10 pF TOPR = -55 to 110° C | VSS = 0 to 42 V ISC = ±26 mA IQ = 1.4 mA Typ. IB = 65 pA Typ. VOS = 10 mV Max IOS = 5 pA Typ. GBW = 5.25 MHz SR = 20 V/µs Typ. CMMR = 100 dB Typ. TA = 0 to 70° C RθJA = 85° C/W | VI = -0.3 to 32 V ICC = 1.4 mA IQ (source) = -10 mA Min IIB = -20 nA Typ. VIO = 3 mV Typ. IOS = ±40 mA B1 = 1.2 MHz SR = 0.5 V/µs Typ. CMRR = 80 dB Typ. TA = 0 to 70° C | VCC = 0 to 22 V ICC = 8 mA Typ. IIO = 10 nA Typ. IIB = 200 nA Typ. VIO = 5 mV Max IOS = 38 mA Typ. B1 = 10 MHz SR = 9 V/µs Typ. CMRR = 100 dB Typ. TA = 0 to 70° C RθJA = 85° C/W |
| OPA2376AQDRQ1 | UA741CP | TLP3553A(F | CD74HC154M96 |
| Dual Rail-to-Rail, Low-Noise, e-trim Op Amp | Gen. Purpose Op Amp w/Offset-voltage Null | SSR SPST-NO Photocoupler/Photorelay | H. Speed CMOS 4-to-16 Decoder/Demultiplexer |
| VS = 7 V Max IIP = -10 to 10 mA IQ = 10 mA Max IB = 0.2 pA Typ. VOS = 5 µV Typ. ISC = 30/-50 mA GBW = 5.5 MHz SR = 2 V/µs Typ. CMRR = 90 dB Typ. TA = -40 to 150° C RθJA = 100.1° C/W | VCC = ±18 V ICC = 1.7 mA Typ. IIO = 20 nA Typ. IIB = 80 nA Typ. VIO = 1 mV Typ. IOS = ±25 mA Typ. GBW = 1 MHz SR = 0.5 V/µs Typ. CMMR = 90 dB Typ. TA = 0 to 70° C RθJA = 87.4° C/W | VDD = 24 V Max IF = 25 mA Max VOFF = 30 V BVS = 2500 V (RMS) VF = 1.64 V Typ. IF = 30 mA Max ION = 4 A Max Ct LED = 70 pF RS Isolation = 1014 Ω PO = 550 mW TOPR = -40 to 110° C | VCC = ±18 V ICC = 1.7 mA Typ. IIO = 20 nA Typ. IIB = 80 nA Typ. VIO = 1 mV Typ. IOS = ±25 mA Typ. GBW = 1 MHz SR = 0.5 V/µs Typ. CMMR = 90 dB Typ. TA = 0 to 70° C RθJA = 87.4° C/W |
| SN7404N | NE555P | CD4093BE | SN74ALS244CN |
| Hex Inverter | Precision Timer/Oscillator | Quad 2-Inp. NAND Schmitt Trigger | Octal Buffer & Line Driver w/3-state Outp. |
| VCC = 4.75 to 5.25 V VIH = 2 V Min VIL = 0.8 V Max IOH = -0.4 mA Max IOL = 16 mA Max VOH = 3.4 V Typ. VOL = 0.2 V Typ. tPLH = 9 ns Typ. tPHL = 10 ns Typ. TA = 0 to 70° C θJA = 80°C/W | VCC = 4.5 to 16 V IO = ±200 mA Max THRESH V. Level at 5V = 2.4 to 4.2 V THRESH Current = 30 nA Typ. TRIG V. Level = 1.1 to 2.2 V Outp.-Pulse Rise/Fall Time = 100 ns Typ. TA = 0 to 70° C RθJA = 85° C/W | VDD = 3 to 18 V IDD = 0.2 µA Typ. CIN = 5 pF Typ. VP Min and Max at 5V = 3.3 V Typ. tPLH = 65 ns Typ. tPHL = 90 ns Typ. tTLH = 40 ns Typ. tTHL = 50 ns Typ. TA = -55 to 125° C θJA = 86°C/W | VCC = 4.5 to 5.5 V ICC (out. High) = 49 mA Typ. VIH = 2 V Min VIL = 0.8 V Max IOH = -15 mA Max IOL = 24 mA Max VOH = 3.2 V Typ. VOL = 0.35 V Typ. tPLH = 10 ns Max tPHL = 10 ns Max TA = 0 to 70° C |
| CD74HC137E | 74HCT03D,653 | TC74HC14APF | ATMEGA164PA-ANR |
| H. Speed CMOS 3-to-8 Decoder/Demux w/Latch | Quad 2-Inp. NAND Gate w/Open Drain Out. | CMOS Hex Schmitt Inverter | 8-bit MCU, 16KB Flash 32 I/O Pins |
| VCC = -0.5 to 7 V ICC = ±50 mA VIH = 3.15 V Min VIL = 1.35 V Max CI = 10 pF Max VOH = 4.4 V Min VOL = 0.1 V Max tPLH, tPHL at 4.5V= 36 ns Max TA = -55 to 125° C θJA = 67°C/W | VCC = -0.5 to 7 V ICC = 50 mA VIH = 2 Min VIL = 0.8 V Max IOZ = ±0.5 mA Max CI = 3.5 pF Max Δt/Δv = 10 ns/V Max tpd = 24 ns Max tt = 7 ns Typ. Tamb = -40 to 125° C Ptot = 500 mW Max | VCC = 2 to 6 V IOUT = ±25 mA VP = 2.75 V Typ. VN = 1.6 V Typ. VH = 1.1 V Typ. CIN = 5 pF Typ. VOH = 4.3 V Typ. VOL = 0.17 V Typ. tPLH, tPHL at 4.5V= 11 ns Typ. Topr = -40 to 85° C PD = 500 mW | VCC = 1.8 to 5.5 V FCLCK = 20 MHz Max Flash = 16 KB SRAM = 1 KB TOP = -40 to 105° C EEPROM = 512 B DC Current per I/O Pin = 40 mA Max RPU (I/O Pull-up Res.) = 20 kΩ Min, 50 kΩ Max Interfaces: UART/USART, I2C, SPI, POR, ADC, WDT |

Note: For the θJA  parameter, when asked about the package in the datasheet, check the datasheet for CD74HC137E, it specifies that, for instance, the PDIP packaged corresponds to E and then the given thermal parameter is given.

Updates:

2025-Jan-13

- Changed all “S” suffixes (seconds) to lower case “s”

- Fixed Δt/Δv units for SN74HC595N and TXS0102DQMR

- Updated diagrams for LM741CN/NOPB, MCP6242-E/P, LM358AN/NOPB, LM386N-1/NOPB, SN74AHCT14N, FODM217DR2V and 140356145400

- Added new components: TL071CP, LM324N, NE5532P, OPA2376AQDRQ1, UA741CP, TLP3553A(F, CD74HC154M96, SN7404N, NE555P, CD4093BE, SN74ALS244CN, CD74HC137E, 74HCT03D,653, TC74HC14APF and ATMEGA164PA-ANR